NEUROGAIT

Biomechanical Running Analysis System

BrainSherpa / NeuroMetric Ecosystem

PROJECT DESIGN DOCUMENT

Version: A1 Prototype

Date: May 2026

Status: Pre-Build — Planning Complete

# 1. Project Overview

NeuroGait is an Android application that connects to a Movesense MD inertial sensor strapped at the sacrum and streams biomechanical running data in real time. It saves each session as a CSV file, displays session history, and allows the user to share data anywhere via the Android share sheet.

NeuroGait is part of the BrainSherpa / NeuroMetric ecosystem, which already includes a working HRV / DFa1 cardiac monitoring app. NeuroGait is being built as a standalone prototype first, with integration into the DFa1 app planned for a future release.

## 1.1 Competitive Context

Primary reference: AletheiaRun (iOS, proprietary sensor, subscription model — aletheia.run). AletheiaRun uses the same sacrum placement concept and the same physics. Their version history shows they started with a simple Force Portrait and added complexity over 12+ months. NeuroGait follows the same incremental strategy.

| Feature | AletheiaRun | NeuroGait |
| --- | --- | --- |
| Platform | iOS only | Android |
| Sensor | Proprietary (locked) | Movesense (open, programmable) |
| Raw CSV export | No | Yes — from day one |
| HRV / DFa1 integration | No | Yes — future roadmap |
| Research-grade output | Limited | Full — open data |
| GPS + IMU fusion | No | Yes — A2 build |

# 2. Hardware

## 2.1 Movesense MD Sensor

- Placement: Sacrum (S1-S2 level, posterior midline, between PSIS landmarks)

- Model: Movesense MD — accelerometer, gyroscope, magnetometer

- NO GPS in the sensor hardware

- NO ECG/HR from sacrum placement — HR requires chest contact

- Connection: Bluetooth LE to Android phone via MDS library (AAR)

- Recommended endpoint: Meas/IMU6/104 (synchronized Acc + Gyro at 104 Hz)

- Accelerometer range: set to ±8g for running (default ±2g clips on hard heel-strike)

## 2.2 Axis Orientation at the Sacrum

When the sensor is mounted flat against the sacrum with the superior pole pointing toward the head:

| Axis | Direction | Biomechanical Meaning |
| --- | --- | --- |
| +Z | Superior (cranial) | Primary step detection axis — vertical oscillation |
| +Y | Left (mediolateral) | Pelvic obliquity and lateral sway |
| +X | Anterior | Sagittal plane motion — fore-aft acceleration |
| GyrX | Roll | Lumbar flexion/extension rhythm |
| GyrY | Pitch | Pelvic drop — Trendelenburg sign |
| GyrZ | Yaw | Pelvic rotation during gait |

## 2.3 Recommended Sample Rates

| Activity | Rate | Endpoint | Notes |
| --- | --- | --- | --- |
| Walking | 52 Hz | IMU6/52 | Saves battery |
| Running | 104 Hz | IMU6/104 | Captures heel-strike peak |
| Sprinting | 208 Hz | IMU6/208 | High BLE bandwidth — test range first |

# 3. Target Device & Project Identity

## 3.1 Test Device

- Phone: Samsung Galaxy A26 5G

- Android version: 16 (One UI 8.0)

- GPS: Standard single-frequency (not dual-band L5)

- Android Studio: Latest version (to confirm on first build session)

## 3.2 App Identity

| Field | Value |
| --- | --- |
| App name | NeuroGait |
| GitHub org | brainsherpa (exact username to confirm) |
| Package name | com.brainsherpa.neurogait OR xyz.neurogait.app (pending domain check) |
| Repo name | neurogait |
| Visibility | Private initially |
| Local folder | Documents/BrainSherpa/NeuroGait/ |

## 3.3 Pending Decisions

- Confirm exact GitHub username (brainsherpa?)

- Check if neurogait.xyz domain is owned — determines package name

# 4. Build Strategy — Prototype First

## 4.1 Why A1 First

AletheiaRun's own release history shows they started simple and built complexity over 12+ months. The correct approach is to see real data from the user's own body before writing algorithms. This prevents building on wrong assumptions.

- Identify which axis is cleanest for step detection at this sacrum

- Calibrate impact g values at actual running pace

- Validate GPS + cadence stride length calculation with real data

- No wasted code on incorrect biomechanical assumptions

## 4.2 A1 Prototype Scope — 4 Screens

### Screen 1 — Connect

- Scan for nearby Movesense devices via BLE

- Tap device name to connect

- Show device serial number when connected

### Screen 2 — Live Stream (the key screen)

Raw values updating in real time:

AccX  AccY  AccZ  (g values — accelerometer)

GyrX  GyrY  GyrZ  (degrees/sec — gyroscope)

Derived values:

Step count

Elapsed time (mm:ss)

Cadence: rolling 10s average (spm)

Peak impact this session (g)

- START / STOP button

### Screen 3 — Post Session

- Session saved confirmation

- CSV auto-saved to phone storage

- Share button appears — user chooses destination (no auto-email)

### Screen 4 — Session History

- List by date/time — matches existing NeuroMetric app style

- Eye icon: view session summary

- Download/share icon: share CSV anywhere via Android share sheet

- Trash icon: delete session

# 5. Metrics — Full Capability Map

## 5.1 Raw Hardware Outputs (Movesense MD at Sacrum)

| Signal | Source | Max Rate |
| --- | --- | --- |
| Acceleration X/Y/Z | Accelerometer | 1666 Hz |
| Rotation X/Y/Z | Gyroscope | 1666 Hz |
| Magnetic field X/Y/Z | Magnetometer | ~50 Hz |
| GPS | NOT available | — |
| Heart rate / ECG | NOT from sacrum | Requires chest |

## 5.2 Derived Metrics — Cadence Group

| Metric | Method | Reliability |
| --- | --- | --- |
| Cadence (steps/min) | Time between Z-axis impact peaks | ★★★★★ |
| Step interval (ms) | Direct from timestamps | ★★★★★ |
| Cadence variability | Std deviation of step intervals | ★★★★★ |
| Rolling cadence (10s) | 10s sliding window mean | ★★★★★ |
| Session avg cadence | Full session mean | ★★★★★ |
| Estimated speed (m/s) | Cadence x stride length model | ★★★☆☆ |
| Real stride length (A2) | GPS speed / (cadence/2) | ★★★★★ |

## 5.3 Derived Metrics — Impact Group

| Metric | Method | Reliability |
| --- | --- | --- |
| Peak impact per step (g) | Resultant acceleration max | ★★★★★ |
| Impact force (Newtons) | peak_g x body_mass x 9.81 | ★★★★☆ |
| Loading rate | Rise rate of g per step | ★★★★☆ |
| Session peak impact | Max g across whole session | ★★★★★ |
| Avg impact per step | Session mean | ★★★★★ |
| Foot strike pattern (A2) | Heel / midfoot / forefoot classification | ★★★☆☆ |

## 5.4 Derived Metrics — Gyroscope / Movement Group

| Metric | Method | Reliability |
| --- | --- | --- |
| Pelvic rotation (deg/step) | GyrZ integrated per step | ★★★★☆ |
| Pelvic lateral tilt (deg) | GyrY — Trendelenburg sway | ★★★★☆ |
| Trunk forward lean | GyrX / pitch angle | ★★★★☆ |
| Sway index | Overall lateral movement magnitude | ★★★★☆ |
| Symmetry score (A2) | Left/right pelvic motion balance | ★★★☆☆ |

# 6. GPS + IMU Sensor Fusion (A2 Feature)

## 6.1 The Key Equation

Combining the phone's GPS speed with the Movesense cadence produces real stride length — no estimation model, no assumptions about leg length.

GPS speed (m/s)  ÷  (cadence_spm ÷ 2 ÷ 60)  =  stride length (m)

Example calculation:

GPS speed:     3.2 m/s

Cadence:       172 spm  →  86 strides/min  →  1.433 strides/sec

Stride length: 3.2 ÷ 1.433  =  2.23 metres per stride

## 6.2 GPS Reliability Rules

| GPS Accuracy | Stride Length Status | UI Indicator |
| --- | --- | --- |
| < 5 metres | CONFIRMED | Green dot — show value |
| 5 – 10 metres | ESTIMATED | Amber dot — show with warning |
| > 10 metres | UNRELIABLE | Red dot — hide or grey out |

## 6.3 Treadmill Mode

- GPS shows zero movement on treadmill — stride length calculation breaks

- Solution: user inputs treadmill speed manually (km/h)

- App uses manual speed in place of GPS speed

- All IMU metrics (cadence, impact, gyro) work perfectly on treadmill

- Session flagged as TREADMILL in CSV and history list

## 6.4 What Real Stride Length Unlocks

| Metric | Calculation | Significance |
| --- | --- | --- |
| Stride length variability | Std dev per 30s window | Injury risk marker |
| Cadence vs stride tradeoff | Speed gain source analysis | Running economy |
| Running economy trend | Stride length drift over session | Fatigue detection |
| Fatigue index | Stride shortening rate — final km | Training load marker |

# 7. User Profile

These are entered once on first launch and persist across all sessions:

| Field | Unit | Purpose |
| --- | --- | --- |
| Name | text | Session labelling |
| Height | cm | Normalisation |
| Weight | kg | Newton force: F = peak_g x kg x 9.81 |
| Bellybutton to ground | cm | Leg length proxy for stride estimation |

Note: Bellybutton-to-ground (standing) is a validated field measurement for leg length in gait research. The actual sacrum (S1) sits approximately 5-8cm posterior and slightly above navel level, but this measurement is the practical standard used in biomechanics literature (Cavanagh model).

# 8. CSV Output Format

## 8.1 A1 CSV — Prototype

timestamp_ms, accX, accY, accZ, gyrX, gyrY, gyrZ,

step_count, cadence_spm, peak_impact_g,

gps_lat, gps_lon, gps_speed_ms, gps_accuracy_m,

stride_length_m

- 104 rows per second (one per IMU sample)

- Named by date/time: NeuroGait_2026-05-03_0923.csv

- Saved to phone external files directory

- Shared via Android share sheet — user chooses destination

- stride_length_m left blank when GPS accuracy > 10m

## 8.2 A2 CSV — Full

+ cadence_10s_rolling, impact_newtons, loading_rate,

+ stride_length_reliable, pelvic_rotation_deg,

+ pelvic_sway_deg, session_mode

session_mode values: OUTDOOR / TREADMILL / INDOOR

# 9. UI Design

## 9.1 Theme

- Dark background — black / near-black

- Accent colour — TBD (open to new palette beyond existing NeuroMetric blue)

- Inspired by: existing NeuroMetric session history screen + AletheiaRun scored dashboard

- Reference apps: Pace Control (image provided) + AletheiaRun (aletheia.run)

## 9.2 Live Session Screen — Option C Hybrid

Top half — Big numbers:

CADENCE          IMPACT           TIME

172            2.8g             04:23

spm            peak            elapsed

Rolling cadence (10s): 174 spm

Est. pace: 5:12 min/km

Bottom half — Live mini score gauges (A2+):

[Stability]    [Symmetry]    [Loading Rate]

arc              arc            arc

74               81             62

## 9.3 Session Detail View (Eye Icon)

- Option selected: BOTH graph over time AND summary numbers

- Charts: cadence timeline, impact timeline, gyro/pelvic timeline

- Force Portrait scatter plots (A2)

- GPS route map coloured by impact force heat map (A2)

## 9.4 Watch Face — BACKLOG

- Wear OS integration — back burner until real outputs validated

- Design: 3 numbers only — Cadence / Impact / Stability

- Colour coded zones: green / amber / red

# 10. Android Studio Project Structure

## 10.1 File Structure

NeuroGait/

├── app/

│   ├── build.gradle

│   ├── src/main/

│   │   ├── AndroidManifest.xml

│   │   └── java/com/brainsherpa/neurogait/

│   │       ├── App.kt                  ← MDS initialisation

│   │       ├── ScanActivity.kt          ← BLE scan & connect

│   │       ├── StreamActivity.kt        ← live data screen

│   │       ├── SessionManager.kt        ← save CSV

│   │       └── HistoryActivity.kt       ← list + share

│   └── src/main/res/layout/

│       ├── activity_scan.xml

│       ├── activity_stream.xml

│       └── activity_history.xml

├── mdslib/                              ← Movesense AAR module

├── build.gradle

└── settings.gradle

## 10.2 Key Dependencies

implementation project(':mdslib')

implementation 'com.polidea.rxandroidble2:rxandroidble:1.17.2'

implementation 'com.google.code.gson:gson:2.10.1'

implementation 'org.jetbrains.kotlinx:kotlinx-coroutines-android:1.7.3'

## 10.3 Key Permissions

BLUETOOTH_SCAN, BLUETOOTH_CONNECT  ← BLE (API 31+)

BLUETOOTH, BLUETOOTH_ADMIN          ← BLE (API < 31)

ACCESS_FINE_LOCATION                ← BLE scan + GPS

WRITE_EXTERNAL_STORAGE              ← CSV save (API < 29)

## 10.4 Movesense API — Key Endpoints

| Endpoint | Description |
| --- | --- |
| Meas/IMU6/104 | Synchronized Acc + Gyro at 104 Hz — USE THIS |
| Meas/Acc/52 | Accelerometer only at 52 Hz |
| Meas/Gyro/52 | Gyroscope only at 52 Hz |
| Meas/IMU9/52 | Acc + Gyro + Magnetometer at 52 Hz |
| PUT Meas/Acc/Config | Set range: {GRange: 8} for running |

# 11. First Build Session — Step by Step

## Before Opening Android Studio

- Create folder: Documents/BrainSherpa/NeuroGait/

- Go to github.com — confirm exact username

- Create new repo: neurogait — Private, Android .gitignore, with README

- Download MDS AAR from: bitbucket.org/movesense/movesense-mobile-lib/downloads/

## In Android Studio

- File > Get from VCS > paste GitHub repo URL

- Point to Documents/BrainSherpa/NeuroGait/ folder > Clone

- File > New > New Module > Import .JAR/.AAR > select mdslib-x.x.x.aar

- Paste build.gradle (Claude provides)

- Paste AndroidManifest.xml (Claude provides)

- Paste each Kotlin file in order (Claude provides one at a time)

- Paste each layout XML (Claude provides)

- Plug in Samsung A26 via USB — Enable USB Debugging

- Press Run — watch it connect to Movesense

# 12. Full Roadmap

| Phase | Name | Key Features |
| --- | --- | --- |
| NOW | A1 Prototype | Connect, stream raw IMU, save CSV, session history |
| NEXT | A2 — Metrics | GPS fusion, real stride length, scored metrics, Force Portrait |
| FUTURE | A3 — Dashboard | Full biomechanics dashboard, route heat maps, AletheiaRun-style scores |
| FUTURE | A4 — Integrate | Merge with NeuroMetric DFa1/HRV — combined sacrum + cardiac session |
| FUTURE | Watch Face | Wear OS — 3 metrics live: Cadence / Impact / Stability |
| FUTURE | AI Coach | Claude API integration — personalised exercise recommendations |

# 13. Key References

| Resource | URL |
| --- | --- |
| Movesense Android Docs | movesense.com/docs/mobile/android/main/ |
| Movesense API Reference | movesense.com/docs/esw/api_reference/ |
| MDS AAR Download | bitbucket.org/movesense/movesense-mobile-lib/downloads/ |
| AletheiaRun competitor | aletheia.run |
| AletheiaRun Force Portraits | aletheia.run/force-portraits |
| AletheiaRun App Store | apps.apple.com/us/app/aletheia-run/id6479916698 |
| GitHub ref: ESSI_APP | github.com/juliaslocke/ESSI_APP |
| GitHub ref: holmmi | github.com/holmmi/Movesense |

Heiderscheit et al. (2011) — Cadence optimal zone 170-180 spm reduces impact loading.

Meardon et al. (2011) — Higher stride variability correlates with stress fracture risk.

Kavanagh & Menz (2008) — Sacrum accelerometer valid for step detection in field conditions.

Riley et al. (2008) — Cadence comparable treadmill vs overground; stride length slightly shorter on treadmill.

Generated from planning sessions with Claude (Anthropic) — May 2026  |  Next update: after A1 first build